# PART 1 – GENERAL

1.1    General Conditions	.1	The General Conditions and Complementary General Conditions clauses are an integral

part of this section and are applied.

1.2    Related Work	.1	Suspended Channel Support for Ceiling-hung Partitions		Section 05500

.2	Rough Carpentry							Section 06100

.3	Shower and Dressing Compartments				Section 10185

.4	Toilet and Bath Accessories					Section 10800

1.3     Samples	.1	Submit duplicate [300 mm x 300 mm] [12" x 12"] samples of panel showing the finish

on both sides, two finished edges and core construction in accordance with Section

01340.

.2	Submit one representative sample of each hardware item,

including brackets, fastenings and trim.

1.4    Shop Drawings	.1	Submit shop drawings in accordance with the requirements of Section 01340.

.2	Clearly indicate production-related details, plans, elevations, and hardware item and installation

details.

1.5    Maintenance data	.1	Provide instructions for maintenance of plastic laminate for incorporation into the

Maintenance and Operation Manual discussed in Section 01730.

1.6    Protection	.1	During shipment and installation, protect all plastic laminate systems including finished laminate

surfaces either by using thick kraft paper or by placing them in cardboard. Store them horizontally at

worksite.

1.7    Warranty	.1	Provide a written guarantee, signed and issued in the name of the Owner stating that the

plastic laminate systems are guaranteed against warping and delamination for a period of

five [5] years (see complete warranty details at www.decolam.ca). Hardware items

are also covered for a period of five [5] years from the date of signature of the Final

Certificate of Completion.

# PART 2 – PRODUCTS

WRITER: retain the desired option in the Materials and Fabrication sections and delete the

others. The option determines the core material used.

OPTION A: Pilasters, doors and separator in solid plastic laminate.

OPTION B: Pilasters and doors in solid plastic laminate, separators in particleboard covered with

plastic laminate.

OPTION C: Pilasters in solid laminate, doors and separators in particleboard covered with plastic

laminate.

WRITER: specify the thickness, colour and pattern, as the cost varies considerably with these criteria. If

more than one sort of plastic laminate sheet is required (colour/finish), specify the various types and ensure

that the drawings and/or specifications indicate in which location(s) each type must be used.

2.1	Materials	.1.1	Decolam Inc. wall-to-wall toilet partitions or equivalent approved

prior to submission of the bid. Colours may be chosen from

among Arborite, Formica, Nevamar, Pionite,

Trespa or Wilsonart products.

## OPTION A

.1.2 	Solid plastic laminate: complies with CAN3-A172-M79, SS category (?),

[19 mm] [3/4”] thick, colour and finish selected by architect/designer.

- OPTION B OR C

.1.2	Solid plastic laminate: complies with CAN3-A172-M79, SS category (?),

[19 mm] [3/4”] thick, colour and finish selected by architect/designer.

.1.3	Plastic laminate sheets: comply with CAN3-A172-M79 general purpose type,

[[1.6 mm] [0.80 mm]] [[1/16”] [1/32”]] thick, colour and finish selected by

architect/designer.

.1.4	Composite doors and/or panels:

.1	Core: particleboard complying with CAN3-0188.1-M78,

Grade H, sanded on both faces, [15.8 mm] [5/8”] thick.

.2	Edge [50 mm] [2”]: solid plastic laminate complying with

CAN3-A172-M79, SS category (?), [15.8 mm] [5/8”] thick.

.2	Laminate adhesives: contact adhesive complying with ONGC 71-GP-20M.

.3 Hardware

.1	Hinges: heavy-duty, corrosion-resistant, wraparound assembly, made of nonferrous

metal (ZAMAC #5) with a satin chrome finish, self-lubricating, double action,

gravity return movement, equipped with a nylon double cam mechanism,

adjustable to hold door open at any angle up to 90°.

.2	Slide bolt and keeper: nonferrous metal (ZAMAC #5) castings	enabling quick

emergency access to toilet compartment by lifting the door.

.3	Doorstop: nonferrous metal (ZAMAC #5) casting with rubber insert.

.4	Wall and connecting brackets: aluminium extruded profiles with clear anodized

finish (optional: may be provided in stainless steel at a higher price).

.5	Coat hook: combination hook and door stop, nonferrous metal (ZAMAC #5)

casting.

.6	Door pulls: of an appropriate J-shaped type on regular outswinging doors and a

D-shaped type for compartments for the handicapped, complying with

CAN/CSA-B651-95 4.2.6.1 and 5.2.2.d, e.

.7  	Overhead brace: extruded aluminium channel with clear anodized finish and

anti-grip design.

.8 	Stainless steel plate: complies with ASTM A167-82, type [[302] [304]] with

brushed finish.

.9 	Fasteners: choice of visible screws: ordinary or tamper-resistant. Ordinary

screws are made of zinc-plated steel and of male female type for hinges.

Tamper-resistant screws are made of Type 410 stainless steel for better

hardness.

.10	Waterproofing material: water-repellent product or adhesive recommended by

the laminate manufacturers.

OPTIONAL

.4	Black polypropylene felt inlaid on both sides of the partition door (privacy

protector or astragal).

  - Fabrication

OPTION A

.1	Make the pilasters, doors and separators from solid plastic laminate panels, [19 mm]

[3/4”] thick, according to the indicated dimensions.

.2	Chamfer exposed edges uniformly at approximately 20°.

OPTION B

.1	Make the pilasters and doors from solid plastic laminate panels, [19 mm] [3/4”] thick,

according to the indicated dimensions.

.2	Make the separators from composite plastic laminate panels with a particleboard core

and a rigid solid laminate edging (frame) of approximately [51 mm] [2”] for a total

thickness of [19 mm] [3/4”], according to the indicated dimensions.

.3	Cement plastic laminate onto the core material. Ensure that the laminate is in contact

over the	entire surface of the core in order to obtain perfect cohesion over the entire

surface area of sheets. This process will entail pressing.

.4	Chamfer exposed edges uniformly at approximately 20°.

OPTION C

.1	Make the pilasters from solid plastic laminate panels, [19 mm] [3/4”] thick,

according to the indicated dimensions.

.2	Make the doors and separators from composite plastic laminate panels with a

particleboard core and a rigid solid laminate edging (frame) of approximately

[51 mm] [2”] for a total thickness of [19 mm] [3/4”], according to the indicated

dimensions.

.3	Cement plastic laminate onto the core material. Ensure that the laminate is in contact

over the	entire surface of the core in order to obtain perfect cohesion over the entire

surface area of sheets. This process will entail pressing.

.4	Chamfer exposed edges uniformly at approximately 20°.

PART 3 – EXECUTION

  - Partition Erection

.1	Install partitions securely, plumb and square.

.2	Leave no more than a 12 mm (1/2") space between wall and panel or end pilaster.

.3	Equip each door with hinges, latch set, and coat hook. Adjust and align hardware items

for smooth, proper operation. Set door open position at 30° to front.

.4	Equip outswinging doors with door pulls.

  - Wall-to-wall partitions

.1 	Fasten pilasters to steel channel supports using pilaster supports.

.2	Cover the suspension beam with plastic laminate having a particleboard core.

.3	      Set bottoms of doors and pilasters so they are level with each other.

Do this with the door closed.

  - Cleaning

.1	After installation, take off all protective items and wash down finished surfaces so they

are free from smudges and stains.

.2	Polish all metal pieces and replace defective or damaged parts.